Xnetdb - A Network Database and Monitoring Tool

Henry Clark

Ohio Supercomputer Center

1224 Kinnear Rd

Columbus, OH  43235

(614) 292-6483

henryc@oar.net

0.  How to get the software

Xnetdb is available via anonymous ftp from thor.oar.net (131.187.1.135) in

the /pub directory.  Note that licensing restrictions are noted in the

file "License" in the distribution.

1.  Xnetdb overview

Xnetdb is a network monitoring tool based on X Windows and SNMP which

also has integrated database and statistic viewing capabilities.  Xnetdb

will determine and display the status of routers and circuits it has

been told to monitor by querying the designated sites and displaying the

result.  Additionally, it also has integrated database functionality in

that it can display additional information about a site or circuit such

as the equipment at the site, the contact person(s) for the site, and

other useful information.  Finally it can gather designated statistical

information about a circuit or router and display it on demand.

2.  Xnetdb configuration files

A.  xnetdb.cf

The configuration file xnetdb.cf specifies all of the things that you want

xnetdb to monitor and/or keep information about.  There are presently three

types of things which can be configured into the file - routers, variables,

and hosts.

A router entry looks like:

ROUTER   <path>   <routername>   <mapname>   <ipaddress>  <community>

The <path> entry specifies the path into the database where xnetdb can find

information about the router.  The <routername> entry specifies the name of

the router in the database.  The <mapname> entry specifies the name which

wil be displayed on the xnetdb map.  The <ipaddress> entry specifies the ip

address of the router.  The <community> entry specifies the snmp community

name to monitor the router under.  An example of a ROUTER entry might be:

ROUTER   Backbone/POP1       RTR1   Router1       1.2.3.4    public

where the <path> variable is set to "Backbone/POP1", the <routername> is

set to "RTR1", the name that will be displayed on the map is "Router1",

and the IP address of the router is "1.2.3.4".  For more information

about the database and how it is structured, see the section covering

the database in detail.

A variable entry looks like:

VARIABLE   <rtraddr>   <variable>   <variabletype>   <community>

The <rtraddr> entry is the ip address of the router.  The <variable> entry

is the name of the snmp variable you wish to monitor.  The <variabletype>

entry is the type of the variable that the agent will return (such as Integer,

IPAddr, etc.).  Finally, <community> is the snmp community name xnetdb will

use when querying the router.  An example of a router entry that would

monitor a default route on a router would be:

VARIABLE  1.2.3.5  1.3.6.1.2.1.4.21.1.1.0.0.0.0   IPAddr  public

Note that as long as the router to be queried (1.2.3.5) returns an

IP address, the query will be held to be valid.  However, if either

the return type is invalid or the query times out, then an error will

be reported by xnetdb.

A host entry looks like:

HOST   <path>   <hostname>   <mapname>   <ipaddress>  <community>

The entires in this configuration statement are exactly the same as in

the ROUTER configuration statement, except for the fact that you are

specifying a host (such as a Sun) instead of a router (such as a cisco).

Xnetdb tries to handle some of the odd things that hosts running assorted

variants of snmp will do, thus the special configuration statement.

B.  location.cf

The location.cf file holds the x and y coordinates of the router on the

xnetdb map.  The format of the file is

<ROUTERNAME> <X> <Y>

where <ROUTERNAME> is the name of the router and <X> and <Y> are the

coordinates of the route on the xnetdb map.  Users should never have

to look at the contents of this file since routers can be moved by

the "point-and-click" method, but the entry can be manually edited

if desired.  Note that if you wish to edit it, you should note the changes

you want to make, stop xnetdb, make the changes, and then restart

xnetdb.

C.  xnetdbquery.cf

The xnetdbquery.cf file holds the addresses of the routers and their

interfaces that you want xnetdbquery to query.  The format of the

file is:

<router-addr>   <interface-addr>  <interface-num>

where <router-addr> is the ip address of the router, <interface-addr>

is the ip address of the interface, and <interface-num> is the snmp

index of the interface.  The user should never edit this file.  It is

automatically created by xnetdb and will change between different

executions of xnetdb.

3.  Xnetdb database configuration

A.  Database Structure

The database which xnetdb uses is designed to be used in a unix

file system and uses the inherent tree structure to convey

meaning to the information.  For example, to find information

for a given site, you descend into the specific directory for

the site and look in the appropriate file to find the information

you're looking for.

The path which xnetdb computes into the database is comprised of

four separate pieces:

1)  The variable "DATABASEPATH" in xnetdb.h.

2)  The variable <path> in xnetdb.cf.

3)  The directory "Routers", "Circuits", or "Contacts" (as needed).

If xnetdb is looking for generic information about the site (such

as the domainname), then it doesn't add anything here.

4)  The variable <routername> in xnetdb.cf.

5)  Finally, the name of the file the information is held in.

A picture of how the pieces look when combined is similar to:

[path to database]               <----- (1)

|

|

[some intermediate path]            <----- (2)

/     |    \                      |

/      |     \                     |

Site1    Site2    Site3             <---

/  |  \

/    |    \

Circuits  Contacts Routers            <----- (3)

|

|

RTR1             <----- (4)

|

|

hardware           <----- (5)

Why so complex you ask?  Well, (1) and (2) could be combined, but

are separated for ease of use.  The directory structure can have

several layers of structure (for example, in our case (1) is two

layers and (2) is three layers) so as to contain information in

just the structure itself.

An example of how xnetdb might construct a path to a file is

lets suppose we wanted to look at the file ("hardware") for

a router so we could get a list of the boards and their rev

levels in that router.  With the following values for all

of the variables:

1)  "DATABASEPATH"  ==  "/usr/db"

2)  "<path>"  ==  "POPS/POP1"

3)  "Routers"

4)  "<routername>"  == "RTR1"

5)  filename == "hardware"

xnetdb would look in "/usr/db/POPS/POP1/Routers/RTR1/hardware" for

information about the boards.

B.  Database Files and Format

An example of the format of the files shown below is given in the

database.examples directory in the xnetdb source directory.

1)  Routers

remoteaccess   --   the remote access number and baud for the router

This line contains the phone number and baud for any out-of-band

access.  An example might be:

(123)456-7890  2400

reportname     --   the name of the router in reports

This file contains "one word", in other words, the name that xnetdb

will search for in the reports it scans.

hardware       --   a list of hardware specific information

This file consists of one line describing the router itself and then

one or more lines consisting of boards in the router.  An example

would be:

RTRNAME  SERIAL  MANUFACTURER MODEL         Rtr1  12345  cisco  ags+

BOARD  REV  SERIAL                          cpu   3   12345

BOARD  REV  SERIAL                          cbus  4   12345

2)  Contacts

primary        --   a list of the primary contacts for the site

secondary      --   a list of the secondary contacts for the site

others         --   a list of any other contacts for the site

The format for all the contact files are the same.  Any line whose

first character is a "#" is treated as a comment.  It then expects

7 lines in the form:

name				John Doe

addr1				The Cow Company

addr2				5 Cow Avenue

city, state zip			Cow City, OH  12345

phone				(123)456-7890

email				cow@cow.com

comment                             Pager: 234-5678

3)  Circuits

csu            --   a list of the csu's on the circuit

The format for this file is one (or two :-)) lines of the form

<place>  <manufacturer>  <model>  <rev>  <serial>

An example would be

RTR1   CowCompany  12321  B1   12345

RTR2   CowCompany  12321  B2   12346

circuit        --   a list of the circuit numbers and carriers

The format for this file is for one or two lines of

<carrier>    <circuitnumber>

where the first line is assumede to be the primary carrier and

the next line is assumed to be a secondary carrier.  An example

would be:

OhioBell     1234567890

CincyBell    0987654321

contacts       --   a list of contacts for the circuit

The format for this file is the same as for the contacts listed

previously.

4)  Sites

domainname     --   the domainname of the site (like oar.net)

An example of a domainname would be:

cis.ohio-state.edu

for the CIS department at OSU.

4.  Xnetdb internal details

A.  Drawing circuits and ethernets

Xnetdb computes where it should place all circuits and ethernets

dynamically via snmp queries while its initializing.  By querying each

interface for both its ip address and its subnet mask, xnetdb can

compute the subnet to which it belongs.  By attaching interfaces

which have common subnets, xnetdb draws circuits and ethernets

(when there's more than one member of the subnet and the speed of

the interfaces are ethernet or the like).  The coordinates of the

circuit (the starting and end points) are derived from the router

to which they're attached.  The coordinates of an ethernet are

computed by attempting to draw a line somewhere in the middle of the

routers to which its attached by averaging the coordinates of the

routes its attached to.

B.  Monitor mode

When xnetdb is started in monitor mode (i.e. with the "-monitor"

option), a separate process is started which actually monitors the

routers which you want to monitor.  The monitor process reads a

configuration file which is created by "xnetdb" which tells it which

routers and interfaces to periodically send a snmp query to and detect

if they are up or down.  The monitor process sends the results of its

querying back via a socket to the xnetdb process, which then can

display the results of the querying (i.e. it can show a line up or

down).  By separating the function of querying from the xnetdb process,

both processes are simplified.

5.  Xnetdb future plans

A.  Traceroute

In a near-future release of xnetdb, I am going to add the capability

of displaying a traceroute graphically.  For example, by clicking on

a router of interest and entering the net which you want to traceroute

to would result in xnetdb displaying the route the packet would take

by turning the circuits involved a different color.

B.  Circuit Stats

Another interesting feature under consideration for xnetdb is the

addition of dynamic statistics gathering capability for circuits.  For

example, an option would be added to the circuit menu to display the

min/avg/max utilization of a circuit or perhaps it could be configured

such that it would print the numbers on the screen next to the circuit.

This is still a bit uncertain as other research is currently being

conducted which is similar to xnetdb and which may be integrated with

it at some point.

C.  Important Hosts

In the near future an option will be added so that important hosts can

be displayed on the xnetdb map and monitored.