From AWOODHULL@hamp.hampshire.edu Sun Aug 14 01:05:07 1994

Received: from hamp.hampshire.edu ([192.33.12.1]) by csd.ia.pw.edu.pl with SMTP id <40159-2>; Sun, 14 Aug 1994 01:03:24 +0200

Date:	Sun, 14 Aug 1994 01:01:00 +0200

From:	Al Woodhull <AWOODHULL@hamp.hampshire.edu>

Subject: Revised ethtest documentation

To:	s133766@ia.pw.edu.pl

Message-id: <D199CBD3655F01807D@hamp.hampshire.edu>

X-Envelope-to: s133766@ia.pw.edu.PL

X-VMS-To: @[.DIS]MARIUSZ

X-VMS-Cc: AWOODHULL

Status: OR

Documentation for Ethtest

Ethtest is part of Mariusz Ostrowski's ethernet driver for Minix.

Documentation by asw, revised 13.08.94

1. Calling Ethtest

Ethtest may be called with a single parameter which will be treated as

the name of a device.

It will respond with this message:

Ethernet frames sender/receiver is using file <devname>

and then will return the ethernet address of the device it is using:

My ethernet address is 00:00:C0:B9:17:1C

Ethtest will then display the main menu:

(s)end, (r)eceive, (i)octl, (q)uit ?

If no parameter was supplied upon calling ethtest it will use /dev/ether

as the default <devname>.

2. Menu options

2.1. (s)end

When (s)end has been selected ethtest prompts for a destination address:

Enter destination

response: enter a six byte ethernet address in hexadecimal with the

bytes separated by colons, i.e., 0:1:2:3:4:5

Ethtest will repeat the "ethernet address:" prompt until a valid six byte

address has been entered.

Ethtest will then prompt for a protocol identifier:

Enter protocol (in little endian):

response: enter a hexadecimal number of up to 4 digits, i.e., 0800. The

only protocol identifiers that are relevant for TCP/IP are 0800 and

0806. These are the identifiers for ethernet packets encapsulating

Internet Protocol (IP)  or Address Resolution Protocol (ARP) packets.

(Note: don't get confused by the "little endian" language, just enter

up to 4 hexadecimal digits in normal most-to-least significant order.)

Ethtest will then display the original and destination addresses and the

protocol identifier:

00:00:C0:B9:17:1C -> 00.01:02:03:04:05, proto 0800

and will then ask for data to send:

Now enter data to send, DEL to stop:

response: enter a line of text; each character typed will be sent after

you press <ENTER>:

The quick brown fox

Ethtest will respond:

Sending 20 bytes of data, (total 34): OK, 34 bytes sent. (As a

matter of fact, 46 bytes are sent in this case. This is because of

Ethernet standard limitation - frame must be >= 60 bytes (14 bytes of

header and  46 bytes of data). If you try to send less than 46 bytes,

the driver still outputs 46 bytes). Following this, ethtest  will

prompt for additional lines of input to be sent until you terminate the

program with the currently defined int character for your terminal

(this is not necessarily DEL). Note that empty lines will not be

accepted, you must press at least one other key before pressing <ENTER>

for each line.

In the (s)end test mode ethtest will never return to the original

(s)end, (r)eceive, (i)octl, (q)uit ?

main menu.

2.2. (r)eceive

The receive test will not work if Tnet is currently active, use the Tnet

halt command before trying the (r)eceive mode of ethtest.

When (r)eceive has been selected ethtest asks:

(b)locking or (n)on-blocking read ?

If you respond with (b)locking ethtest will wait to be interrupted upon

receipt of a packet. This corresponds to the normal mode of operation

of Tnet. In (n)on-blocking mode ethtest will poll the ethernet card

continuously. Although network software won't normally act this way

this provides a method of determining whether interrupt conflicts or

mis-assignments are responsible for network problems.

display data ?

response: y or n

You will then see the following message:

Receiving frames, press DEL to stop

(note that if stty has been used to change the int character for your

terminal you must use the currently defined int character, not DEL, to

stop.)

If you have selected non-blocking mode you will see a display:

no frame, looping: <n>

where n continuously increases, counting loop iterations to let you

know that it's working.

See section 3, below, for examples of the packet data display.

In the (r)eceive test mode ethtest will never return to the original

(s)end, (r)eceive, (i)octl, (q)uit ?

main menu.

2.3. (i)octl

The (i)octl mode allows you to check and set some parameters before

proceeding to  the (s)end or (r)eceive tests.

Your choices are:

(1) get address, (2) get filter, (3) set filter ?

Upon selecting (1) you will see a message like:

my ethernet address is 00:00:C0:B9:17:1C

Upon selecting (2) you will probably see:

number of filters = 2, mode = 00   0800 0806

This is the default configuration. Mode 00 is "normal", in which only

the selected protocol types are accepted. 0800 and 0806 are the

identifiers for ethernet packets encapsulating Internet Protocol (IP)

or Address Resolution Protocol (ARP) packets, which are the only

packet types relevant to Tnet or other TCP/IP systems.

If you select (3) you will be asked:

number of filters (0..4) ?

If you reply with a non-zero number you will be further asked:

(n)ormal (i.e. accept) or (r)everse (i.e. reject) mode ?

and then prompted for a mode (up to 4 hexadecimal digits) for each

filter:

filters[1] =

filters[2] =

etc...

Finally you will see this message:

new filters set

and then you will be returned to the

(s)end, (r)eceive, (i)octl, (q)uit ?

main menu.

3. Packet dump format

The majority of the packets you will see will look like this:

00:AA:00:0D:2D:2D -> FF:FF:FF:FF:FF:FF, proto 0806, data size 46

<data>

----------------     -------------------      ----

sender address       destination address      type

This is a broadcast packet, as indicated by the FF:FF:FF:FF:FF:FF

address. Type 0806 packets (Address Resolution Protocol, ARP) are used

to discover the ethernet address of a destination, and so are broadcast

to all nodes.

Ethtest is capable of displaying the contents of a packet as a string,

but with an ARP packet no printable bytes are encountered before a zero

byte, which the C program interprets as the end of the string.

(A good exercise: modify the eth_dump function in ethtest to display

the contents of a packet in hexadecimal and ASCII. Hint: you'll find

such code in Tnet's ether.drv module.)

On an ethernet that supports other protocols you may see different

packet types. For example:

02:07:01:0F:F4:4A -> FF:FF:FF:FF:FF:FF, proto 0BAD, data size 46

Q1

This is a Banyan Vines packet. This packet apparently serves a similar

function to the TCP/IP ARP packet, since it is being broadcast to all

nodes. Note is this case part of the data appears as the ASCII string

"Q1".

The ethernet interface will only respond to packets to its own address

or to a broadcast (FF:FF:FF:FF:FF:FF) address, so you are likely to see

only broadcast packets when using ethtest, and in a TCP/IP system these

will probably be mostly ARP packets. If you want to see packets

addressed to your own node you can either use another copy of ethtest

or a similar program on another node to send packets, or you can first

start Tnet, start a telnet or ftp session with another node, and then

halt Tnet (remember, ethtest won't receive while Tnet is active) and

start ethtest.

The ethernet hardware can also receive multicast frames. However, this

ethernet driver software cannot.

4. Error messages:

These are all the error messages in the source code.

4.1. calling errors:

These errors may be seen if you call ethtest with a device name

parameter or if the default device /dev/ether doesn't exist:

could not open <devname> for reading

meaning: device doesn't exist or has inadequate permissions

IOCTL call (EIOCGETADDR) failed

meaning: local ethernet address could not be read. Possibly device is

not of the right kind.

4.2. (s)end errors

could not open <devname> for writing

meaning: device doesn't exist or has inadequate permissions

error, returned <n> meaning: transmission failed. This usually

means two processes (on different nodes) tried to send at the same

time.

4.3. (r)eceive errors

could not open <devname> for reading

meaning: device doesn't exist or has inadequate permissions

error reading <devname>

meaning: Probably collision of packets, caused by more than one node

transmitting at once.

strange: ret==0

strange: received <n> bytes

meaning: these  messages are returned if packets are detected that are

smaller than the smallest legal ethernet packet. The error may be

elsewhere in the network or due to mangled data caused by packet

collisions.

4.4. (i)octl errors

could not open <devname> for reading

meaning: device doesn't exist or has inadequate permissions

IOCTL call (EIOCGETADDR) failed

meaning: couldn't read the ethernet address of the local interface

IOCTL call (EIOCGETFILTER) failed

meaning: couldn't read interface current filter settings

IOCTL call (EIOCSETFILTER) failed

meaning: couldn't change interface filter settings.